ReadMe

This document is provided to support the dNBR classification of the Rwenzori 2012 wildfire.

Users can open the dNBR_Uganda QGIS file to see the final figure output.

Burn Area Delineation and Burn Severity Classification

The wildfire burned area delineation was determined using The Normalised Burn Ratio (NBR; Key & Benson, 2006). The NBR integrates the two spectral bands which respond most to burning, TM/ETM+ Band 4 (B4, near infrared reflectance) and Band 7 (B7, shortwave infrared reflectance), and is calculated for each pixel in the Landsat image using the equation:

NBR = (B4 – B7) / (B4 + B7)

Where B is the reflectance per pixel corrected for atmospheric transmittance (Key & Benson, 2006).  The difference (dNBR) between pre- and post-fire images from 24th January 2012 and 12th March 2012 provides a measure of burn severity on a relative USGS-defined scale (Key & Benson, 2006).

Due to the failure of the Scan Line Corrector on the Landsat 7 satellite from 2003, images from this period contains stripes which run through the images. These were filled for the final dNBR classification file using the raster fill nodata tool in QGIS.

The histogram tool was used to determine the % of pixels within each dNBR classification.

Files used for analysis are available for download from the USGS Earth Explorer:

Pre_B4: LE07_L1TP_173060_20120124_20200909_02_T1_B4

Pre_B7: LE07_L1TP_173060_20120124_20200909_02_T1_B7

Post_B4: LE07_L1TP_173060_20120312_20200909_02_T1_B4

Post_B7: LE07_L1TP_173060_20120312_20200909_02_T1_B7